Audio and Signal Processing

2026311323 Howon Chung

1. Objective

Phase-derivative-based phase unwrapping predicts the next unwrapped phase value and resolves the remaining integer ambiguity q(w_k) by minimizing an error term between that prediction and the candidate wrapped-phase values.

The minimizing q(w_k) is derived below by setting the derivative of the squared error to zero and solving, then implemented and validated on both a synthetic and a real speech signal.

2. Setup

Measured phase (principal value, PV) is restricted to (-pi, pi] by atan2, so the true phase appears to jump whenever it crosses that range (wrapping).

Unwrapped phase at bin k: theta(w_k) = PV[X(w_k)] + 2*pi*q(w_k), for some integer q(w_k).

theta_hat(w_k) is predicted by numerically integrating the signal's own phase derivative (trapezoidal rule) from the already-unwrapped theta(w_{k-1}).

Remaining problem: given PV[X(w_k)] and the prediction theta_hat(w_k), find the integer q(w_k).

3. Deriving q(w_k)

Define the squared error: E(w_k)^2 = (PV[X(w_k)] + 2*pi*q - theta_hat(w_k))^2, treating q as a continuous real variable for the moment.

Differentiate with respect to q and set it to zero: d(E^2)/dq = 2 * 2*pi * (PV[X(w_k)] + 2*pi*q - theta_hat(w_k)) = 0.

Since 2*2*pi != 0, the bracketed term itself must be zero: PV[X(w_k)] + 2*pi*q - theta_hat(w_k) = 0.

Solving for q: q* = (theta_hat(w_k) - PV[X(w_k)]) / (2*pi).

q* is generally not an integer (it only has to be exact if the prediction is perfect). Because q(w_k) is defined as an integer, the actual choice is the nearest integer to q*: q(w_k) = round(q*).

E(w_k) is naturally an absolute-value error, so minimizing the squared error and then rounding to the nearest integer is equivalent to minimizing the absolute error over the integers directly.

4. Implementation

Modulo-2pi method (a simpler baseline): adjacent-bin phase difference compared against a threshold (2*pi - eps); corrected by +-2*pi if exceeded.

Derivative-based method: phase derivative computed analytically from the signal (DTFT derivative identity), integrated via the trapezoidal rule, q(w_k) resolved by the rounding formula derived above.

5. Validation 1: Synthetic Signal (Pure Delay)

Test signal: x[n] = delta[n-7] (true phase = -7*w, a straight line).

| Method | Max error vs. true phase |
| --- | --- |
| Derivative-based (derived formula) | 1.8e-15 (essentially exact) |
| Modulo, eps = 1e-6 | 18.850 rad (~3 full cycles, fails) |
| Modulo, eps = 0.1 | 1.8e-15 (essentially exact) |

Cause of the eps=1e-6 failure: a real wrap's jump is 2*pi minus whatever small amount the true phase moved between samples, never exactly 2*pi.

An eps this close to zero pushes the threshold (2*pi - eps) almost all the way to 2*pi itself, stricter than the real jumps it is supposed to catch.

6. Validation 2: Real Recorded Speech (Voiced Frame, 25 ms)

numpy.unwrap used as an independent reference.

| Method | Max error vs. np.unwrap |
| --- | --- |
| Modulo, eps = 0.1 | 289.0 rad (~46 full cycles) |
| Derivative-based (derived formula) | 1.7e-13 (essentially exact) |

Cause: this frame's spectrum has a zero close to the unit circle, so the phase derivative spikes to about 1,002 at one frequency.

Adjacent-bin phase jumps exceeded pi at 50 points, but none exceeded 2*pi-0.1 (~6.18) -- the modulo method missed every one of them.

The derivative-based method, using the derived q, remained accurate on real speech despite this.

7. Conclusion

The rounding rule q(w_k) = round((theta_hat - PV) / (2*pi)), obtained by minimizing the squared prediction error, is the correct way to resolve the remaining integer ambiguity.

The modulo method depends on adjacent-bin phase change staying small, which holds for a clean synthetic signal but breaks down on real speech near strong resonances.

The derivative-based method, using the derived q formula, does not rely on that assumption and stayed accurate in both cases.